DESIGN FORGE

PCB DESIGN AND GERBER GENERATION

COMMON EMITTER AMPLIFIER

Design and Creation of PCB Layout, Track Routing and Gerber Files

A 10-Page Technical Report

Submitted by

P. Gokulnath

Department of Electronics and Communication Engineering

Academic Year 2026

PCB Design Software: KiCad

Design type: Single-layer prototype PCB

# 1. Abstract

This report presents the complete printed circuit board design procedure for a single-stage Common Emitter (CE) BJT amplifier. The work begins with the electrical schematic, continues through component assignment and PCB footprint selection, and concludes with placement, track routing, design-rule checking and generation of fabrication-ready Gerber files. The amplifier is designed around an NPN transistor and a voltage-divider bias network so that a small AC input can be amplified with a stable operating point.

The PCB is intentionally designed as a compact educational prototype. Signal paths are kept short, power and ground paths are made wider than small-signal traces, and polarized components are oriented consistently to reduce assembly errors. The final fabrication package contains the copper, solder-mask, silkscreen, board-outline and drill information normally required by a PCB manufacturer.

# 1.1 Objectives

Create the CE amplifier schematic and verify electrical connectivity.

Assign practical through-hole footprints and define a compact PCB outline.

Place components to minimize unwanted coupling and simplify assembly.

Route PCB tracks with suitable widths, clearances and return paths.

Run design-rule checks and correct routing/clearance errors.

Generate Gerber and drill files suitable for fabrication and inspection.

# 1.2 Expected Outcome

The expected outcome is a functional low-frequency voltage amplifier PCB that can be assembled using commonly available components. The report also documents the design decisions so that the same workflow can be repeated for other analog amplifier circuits.

# 2. Circuit Description and Design Requirements

# 2.1 Common Emitter Configuration

A common emitter amplifier uses the emitter as the common reference node for the input and output signal paths. In the proposed circuit, the input is coupled through C1 to the transistor base, while the output is taken from the collector through C2. R1 and R2 form the base voltage-divider, RC provides the collector load, and RE stabilizes the emitter current. C3 bypasses the emitter resistor for AC signals, increasing voltage gain while retaining DC bias stability.

# 2.2 Proposed Component Values

| Ref. | Component | Value | Footprint |
| --- | --- | --- | --- |
| Q1 | NPN transistor | BC547 | TO-92 THT |
| R1 | Resistor | 47 kΩ | Axial DIN0207 |
| R2 | Resistor | 10 kΩ | Axial DIN0207 |
| RC | Collector resistor | 2.2 kΩ | Axial DIN0207 |
| RE | Emitter resistor | 1 kΩ | Axial DIN0207 |
| C1 | Input coupling capacitor | 10 µF | Radial THT |
| C2 | Output coupling capacitor | 10 µF | Radial THT |
| C3 | Emitter bypass capacitor | 100 µF | Radial THT |
| J1 | Input connector | 2-pin | 2.54 mm |
| J2 | Output connector | 2-pin | 2.54 mm |
| J3 | Power connector | 2-pin | 2.54 mm |

# 2.3 Electrical Design Considerations

The divider values establish the base DC voltage, while RE converts base-voltage changes into emitter-current stabilization. The collector resistor converts transistor current variations into an output voltage. Coupling capacitors block DC from external source and load circuits. The practical PCB should provide a clearly marked VCC and GND connection and enough space around Q1 to identify its flat side and pin order.

# 2.4 Typical Operating Assumptions

| Parameter | Design target |
| --- | --- |
| Supply | 9 V DC nominal |
| Signal type | Small-signal sine wave |
| Target use | Audio / low-frequency demonstration |
| PCB technology | FR-4, 1-layer copper prototype |
| Copper | 1 oz nominal |
| Minimum track width | 0.50 mm signal; 0.80–1.00 mm supply |
| Suggested clearance | 0.25 mm or greater |

# 3. Schematic Capture and Footprint Assignment

# 3.1 Schematic Workflow

The schematic is first entered in KiCad Schematic Editor. Each component receives a reference designator and value. Net labels are used where useful, especially for VCC, GND, INPUT and OUTPUT. After the schematic is electrically complete, footprints are assigned according to component package and the intended assembly method.

Place Q1 at the center of the amplifier signal path.

Use R1/R2 close to the base node to reduce the length of the bias network.

Keep C1 close to the transistor base for a short input path.

Place RC close to Q1 collector and the supply rail.

Place RE and C3 near the emitter node.

Place J1, J2 and J3 at the board edge for easy probing and wiring.

# 3.2 Net Connectivity

| Net | Main connections | Purpose |
| --- | --- | --- |
| VCC | J3+, RC, R1 | DC supply |
| GND | J3−, R2, RE return, C3− | Reference/return |
| VIN | J1+, C1 input | Amplifier input |
| BASE | C1, R1, R2, Q1 base | Bias/input node |
| COLLECTOR | Q1 collector, RC, C2 | Amplified signal node |
| EMITTER | Q1 emitter, RE, C3 | Bias stabilization |
| VOUT | C2, J2 output | Amplifier output |

# 3.3 Footprint Rules

Through-hole footprints are selected because they are inexpensive, easy to solder manually and suitable for an academic prototype. Resistor footprints use standard axial spacing, while electrolytic capacitors use radial footprints with clearly differentiated positive and negative pads. Q1 must use the correct BC547 footprint or an equivalent footprint matching the exact transistor pin arrangement purchased.

# 3.4 Electrical Verification

Before PCB layout, the schematic should pass the electrical rules checker. Any unconnected pins, duplicated references, missing footprints or accidental power connections must be corrected before transferring the design to the PCB editor. This stage prevents many layout errors from becoming physical board errors.

# 4. PCB Board Setup and Component Placement

# 4.1 Board Outline

A compact rectangular outline of approximately 60 mm × 40 mm is suitable for the proposed circuit. The exact dimensions may be changed to match the fabrication service. The board edge is drawn on the fabrication/board-outline layer and must form a closed profile.

# 4.2 Placement Strategy

Component placement strongly influences analog performance. The input connector is kept away from the output connector where practical, and the transistor is placed near the center so the input, collector and emitter networks remain compact. Power entry is placed near the supply-side circuitry. Components are aligned consistently to improve readability and assembly.

| Placement item | Recommended position / rule |
| --- | --- |
| J1 Input | Left board edge |
| Q1 | Central area |
| R1/R2 | Near Q1 base |
| C1 | Between J1 and Q1 base |
| RC | Between VCC and Q1 collector |
| RE/C3 | Near Q1 emitter |
| C2 | Between collector and J2 |
| J2 Output | Right board edge |
| J3 Power | Top/bottom edge near VCC/GND |

# 4.3 Analog Layout Precautions

Avoid routing the output trace parallel to the input trace for long distances.

Keep the high-impedance base node physically short.

Use a clean ground return and avoid unnecessary loops.

Keep decoupling close to the power-entry/transistor supply region.

Provide silkscreen polarity marks for electrolytic capacitors.

Leave enough copper clearance around mounting holes and the board edge.

# 4.4 Design Appearance

The proposed layout uses a clean educational style: reference designators are visible, connectors face outward, polarized components have obvious polarity, and the routing is organized around the signal flow from input to transistor to output.

# 5. PCB Track Routing

# 5.1 Routing Procedure

After placement, the PCB editor imports the ratsnest showing the required connections. Routing begins with critical signal nets and then proceeds to supply and ground connections. Since this is a single-layer board, traces should be arranged to avoid crossing. If an unavoidable crossing occurs in a prototype, a zero-ohm link or wire link can be considered, although the preferred final design is a completely routable PCB.

# 5.2 Track Width and Clearance

| Net class | Suggested width | Typical use |
| --- | --- | --- |
| SIGNAL | 0.50 mm | Input, base, collector, output |
| POWER | 0.80–1.00 mm | VCC and main return |
| GROUND | 0.80–1.00 mm | High-confidence low-impedance return |
| Clearance | ≥0.25 mm | Copper-to-copper separation |

# 5.3 Routing Order

Route the short transistor-to-resistor connections first.

Route the sensitive base/input network next.

Route the collector/output network while keeping it away from the input.

Route VCC with a wider track.

Complete the ground return with the shortest practical path.

Inspect every ratsnest line until all connections are physically routed.

# 5.4 Ground Strategy

For a small single-stage amplifier, a wide ground trace or compact ground copper region can reduce return-path impedance. If a copper pour is used, it must be assigned to GND and checked after filling zones. The final Gerber must be inspected to ensure the intended copper region is actually included.

# 5.5 Noise and Stability

Analog amplifier boards can suffer from unwanted feedback when input and output traces are placed close together. The routing therefore follows the signal direction and minimizes long parallel runs. Supply decoupling and a low-impedance return path also help prevent supply-related disturbances from entering the signal path.

# 6. Design Rule Check and PCB Verification

# 6.1 Design Rule Check (DRC)

The DRC is performed after routing and copper-zone filling. It checks geometric and electrical constraints such as track clearance, unconnected nets, board-edge violations, hole clearances and other fabrication rules. The exact values should be updated to match the PCB manufacturer's capabilities.

| Check | Expected result | Action if failed |
| --- | --- | --- |
| Unconnected nets | 0 | Route missing connection |
| Clearance violations | 0 | Move or reroute trace |
| Board-edge violations | 0 | Move copper/components inward |
| Hole/annular-ring issues | 0 | Use suitable footprint |
| Silkscreen overlap | 0 or acceptable | Move reference text |
| Footprint mismatch | 0 | Verify component package |

# 6.2 Visual Inspection Checklist

Confirm Q1 orientation and pin mapping.

Confirm capacitor polarity.

Confirm J3 VCC/GND labels.

Confirm J1 input and J2 output labels.

Check that no track is accidentally connected to a neighboring pad.

Check board outline dimensions.

Check mounting-hole clearances if mounting holes are used.

Inspect the filled ground region and isolated copper islands.

# 6.3 Fabrication Readiness

A board is fabrication-ready only after both electrical and visual checks are complete. It is good practice to open the generated Gerber files in an independent Gerber viewer and compare the displayed layers with the PCB editor. This catches incorrect layer selection, missing board outline information or unexpected plot settings.

# 6.4 Assembly and Test Plan

After fabrication, solder components according to the reference designators. Inspect solder joints, check resistance between VCC and GND before applying power, and then apply a current-limited supply. A small sine-wave input can be applied using a function generator or audio source, and the output can be observed on an oscilloscope.

# 7. Gerber File Generation

# 7.1 Required Fabrication Outputs

Gerber files are generated from the completed PCB layout. For a simple one-layer through-hole board, the essential fabrication information includes the copper layer, solder mask where applicable, silkscreen and board outline. Drill information is normally supplied as an Excellon drill file.

| File / layer | Purpose |
| --- | --- |
| F.Cu / copper Gerber | Top-side copper pattern |
| B.Cu / copper Gerber | Bottom copper if used; not required for true single-layer routing |
| F.Mask | Top solder-mask opening data |
| B.Mask | Bottom solder-mask data if applicable |
| F.Silkscreen | Top component labels and markings |
| Edge.Cuts | Board outline / fabrication profile |
| Excellon drill | Through-hole drilling locations and sizes |

# 7.2 Gerber Plot Procedure in KiCad

Open the PCB layout in PCB Editor.

Run the board design-rule check and resolve required errors.

Open the Gerber Plot/Plot dialog.

Select the fabrication layers required by the manufacturer.

Enable the board outline layer.

Generate drill files using the appropriate drill settings.

Generate the Gerber files into a dedicated fabrication folder.

Open the files in a Gerber viewer and inspect each layer.

# 7.3 Suggested Gerber Package

A typical output folder can contain files similar to: CommonEmitter-F_Cu.gbr, CommonEmitter-F_Mask.gbr, CommonEmitter-F_Silkscreen.gbr, CommonEmitter-Edge_Cuts.gm1 and CommonEmitter-PTH.drl. Exact extensions and naming depend on KiCad version and plot settings.

# 7.4 Important Note

The filenames above are representative examples of the intended fabrication package. They are not presented as actual generated manufacturing files unless the PCB layout has been created and plotted in KiCad. The report therefore documents the correct generation procedure and expected output structure.

# 8. PCB Manufacturing, Assembly and Testing

# 8.1 Fabrication Preparation

Before sending the design for fabrication, confirm board thickness, copper weight, surface finish, solder-mask availability, minimum drill size and manufacturer's minimum track/clearance capability. For an educational prototype, a standard FR-4 board with approximately 1.6 mm thickness is commonly suitable.

# 8.2 Assembly Sequence

Inspect the bare PCB and confirm the board outline.

Populate resistors first because their low height makes later soldering easier.

Install Q1 while matching its package orientation and pinout.

Install electrolytic capacitors with correct polarity.

Install connectors and mechanically secure them.

Inspect solder bridges and cold joints under magnification.

Clean the PCB if required by the soldering process.

# 8.3 Pre-Power Electrical Test

| Test | Expected observation |
| --- | --- |
| VCC to GND resistance | No near-short circuit |
| Visual polarity check | All electrolytic capacitors correct |
| Q1 orientation | Matches selected footprint/pinout |
| Connector continuity | Pins match labels |
| Ground continuity | Common ground path present |

# 8.4 Functional Test

Apply a regulated 9 V supply and verify the transistor DC operating voltages. Then apply a low-amplitude sine wave to the input. Observe the output waveform and compare input and output amplitudes. The CE stage should provide voltage gain within the limits set by the bias point, transistor characteristics, emitter bypassing and load.

# 8.5 Expected Waveform

For a small sinusoidal input, the collector output is approximately sinusoidal but is inverted in phase relative to the input. If the input amplitude is increased too much, clipping can occur because the transistor leaves its intended active region. Such clipping is useful as a diagnostic sign that the signal level or bias point needs adjustment.

# 9. Results, Conclusion and References

# 9.1 Result Summary

| Design stage | Result / status |
| --- | --- |
| Schematic capture | Completed design procedure |
| Footprint assignment | Through-hole footprints selected |
| Component placement | Signal-flow-oriented placement |
| PCB routing | Single-layer routing strategy defined |
| DRC | Must be verified as zero critical violations |
| Gerber plotting | Fabrication layers and drill files defined |
| Assembly test | Input/output and DC checks specified |

# 9.2 Advantages of the Proposed PCB

Simple and low-cost single-stage amplifier architecture.

Through-hole components make manual assembly straightforward.

Compact routing reduces unnecessary signal-loop area.

Clearly labelled connectors simplify laboratory testing.

Gerber and drill outputs can be directly checked before fabrication.

# 9.3 Conclusion

The Common Emitter Amplifier PCB provides a complete example of the transition from an analog circuit schematic to a manufacturable printed circuit board. The design process emphasizes correct footprint selection, careful analog component placement, controlled routing, ground-return planning, design-rule verification and independent Gerber inspection. Following these stages reduces the probability of electrical, mechanical and fabrication errors. The resulting fabrication package can be used to manufacture the board once the final KiCad project is physically routed and the generated Gerbers are verified.

# 9.4 References

1. KiCad Documentation, PCB Editor and Gerber Plotting documentation.

2. Paul Horowitz and Winfield Hill, The Art of Electronics, 3rd Edition, Cambridge University Press.

3. Adel S. Sedra and Kenneth C. Smith, Microelectronic Circuits, Oxford University Press.

4. Thomas L. Floyd, Electronic Devices, Pearson.

5. BC547 NPN transistor manufacturer datasheet and package/pinout documentation.

## Appendix: Recommended Final File Set

KiCad project files should include the schematic, PCB layout and project configuration. The fabrication folder should contain the plotted Gerbers, drill file and, if required by the manufacturer, a fabrication drawing or readme specifying board thickness, copper weight and special requirements.